Duo-Gard Industries Inc.

40442 Koppernick Road

Canton, Michigan  48187

Phone    (734) 207-9700

Fax    (734) 207-7995

www.duo-gard.com

Section 10 22 00

Interior Specialties – Partitions

Series 1500 –Continuous Interior Translucent Wall System

40mm–Stremeline

PART 1	GENERAL

1.1	SECTION INCLUDES

A.     Translucent Interior Wall System.

  - RELATED SECTIONS

A.	Section 08 84 00 - Translucent Plastic Glazing.

1.3	REFERENCES

- ASTM E 84 - Surface Burning Characteristics of Building Materials.

- ASTM D 635 - Rate of Burning and/or Extent and Time of Burning of Self-Supporting Plastics in a Horizontal Position.

- ASTM D 1929 – Ignition Temperature of Plastics.

- ASTM D 2843 – Density of Smoke.

1.4	DESIGN REQUIREMENTS

A.	Vertical loading performance commensurate with usage.

- Maximum Allowable Deflection of Structural Members:  Maximum of L/120 of clear span.

1.5	SUBMITTALS

A.	Comply with Section 01 33 00 - Submittal Procedures.

B.	Product Data:  Submit manufacturer's product data, fire testing, load calculations, including materials, components, fabrication, finish, and installation instructions.

C.	Shop Drawings:  Submit manufacturer's shop drawings, including plans, elevations, sections, and details, indicating dimensions, tolerances, profiles, anchorage, connections, fasteners, hardware, provisions for expansion and contraction, flashing, finish, and attachments to supports of glazing, framing, and options.

D.	Samples:  Submit manufacturer’s samples for each glazing type, framing system, finish, and color specified.

E.	Manufacturer’s Certification:  Submit manufacturer’s certification that materials comply with specified requirements and are suitable for intended application.

F.	Manufacturer’s Project References:  Submit list of completed projects including project name and location, name of Architect or Interior Designer and type of daylighting manufactured.

- Warranty:  Submit manufacturer's standard warranty.

- Testing Reports:  Submit manufacturer’s test reports.

  - Fire tests

  - Air infiltration test.

  - Water penetration test.

1.6	QUALITY ASSURANCE

A.	Manufacturer’s Qualifications:

1.	Continuously engaged in translucent insulated daylighting manufacturing with a minimum of 10 years successful experience.

2.	Able to demonstrate successful performance on comparable projects.

3.	Responsible for all components, including structural design.

B.	Installer’s Qualifications:

1.	Authorized by manufacturer to install translucent glazing products.

- Trained by manufacturer’s standard training methods and policies.

C.	Approved Manufacturers:

All manufacturers acceptable for use on this project must be approved prior to bid. Manufacturers must submit evidence of compliance with all performance criteria specified including test reports.  Any exceptions taken from this specification must be noted on the approval request.  If approval is given, product performance will be as specified.  Should non-compliance be subsequently discovered, the previously given approval will be invalidated and use of the product on the project will be disallowed.  Requests for approval, with all appropriate submittal data and samples must be received no less than 15 days prior to bid date.  A list of all approved manufacturers and products will be issued by addendum. No other manufacturers will be acceptable.  No verbal approval will be given.

1.7	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer's original, unopened containers and packaging, with labels clearly identifying product name, manufacturer, and location of installation.

B.	Storage:

1.	Store materials in a clean, dry area indoors in accordance with manufacturer's instructions.

2.	Keep temporary protective coverings in place.

3.	Do not expose panels to direct sunlight for extended periods.

1.8	WARRANTY

A.	Warranty Period:  Ten years on weatherization starting on date of substantial completion.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	Duo-Gard Industries Inc., 40442 Koppernick Road, Canton, Michigan 48187.

Phone (734) 207-9700.  Fax (734) 207-7995.  Web Site www.duo-gard.com.

2.2	GLAZING

- Product:  Series 1500 Translucent continuous interior wall system glazed with

polycarbonate structured sheet.

- Panel Thickness:  40 mm.

C.	Panel Profile:  7-wall tongue and groove

D.	Color: manufacturer’s standard -clear, opal.

(other colors available, please contact Duo-Gard)

E.	U-Value, ASTM C 236:  0.29

- Light Transmission, ASTM D 1003:  49 percent (clear)

30 percent (opal)

G.	Fire Tests:

  - Flame Spread, ASTM E 84: Class A.

  - Smoke Density, ASTM E 84: Class A.

  - Smoke Developed, ASTM D 635:  CC1

  - Ignition Temperature, ASTM D1929

  - Density of smoke, ASTM D 2843

H.	Panel widths: 19.7 inches.

I. 	Sheets are connected together with tongue and groove joint. No mullion allowed.

J.	Glazing Options:  one side of wall area.

both sides of wall area.

2.3	STRUCTURAL FRAMING SYSTEM

A.	Framing System:  Series 1500 Snap Cap and Base Glazing Channel System.

- Alloy:  6005 - T5

B.	Combined Maximum Deflection:  1 inch.

C.	Provide steel stud structure at panel’s vertical center dimension (by contractor) or

aluminum structural frame per project drawings (by translucent panel manufacturer).

D.	Glazing system required to span from head to sill in one continuous panel.

2.4	MATERIALS

A.	Glazing Panel:

1.	Panel:  Polycarbonate structured sheet.

2.	UV Stabilization:  Coextruded into panel, not coated.

3.	Resist Yellowing, Maximum 10 delta for a minimum of 10 years.

- Appearance:  Uniform in color.

- Expansion and Contraction:  Design and install components with provisions for expansion and contraction.

- No caulk or wet seals are to be used on glazing panel.

- Produced:  USA certificate of origin required.  Panels produced outside of USA will not be allowed.

B.	Joint Sealant:

1.	Factory-Applied Sealant:  Gunnable, nonhardening, elastomeric sealant.  ASTM C 920, Type S, Class 12, Grade NS.  Fed Spec TT-S-1657, Type 1.n.

2.	Field-Applied Sealant:  Approved by translucent insulated daylighting manufacturer.  As specified elsewhere in specifications.

C.	Field Fasteners:

- Comply with translucent insulated daylighting manufacturer’s instructions for fastener types, quantities, and usage.

- Cadmium-plated or better.  Prevent oxidation or electrolytic interaction with framing.

3.	Aluminum-to-Aluminum Connections:  Self-drilling screws, No. 10 and No. 12, of sufficient length for full-thread engagement, as determined by manufacturer.

2.5	COLOR AND FINISH

A.	Panel Color:

1.	Manufacturer's standard-clear, opal.

(other colors are available, please contact Duo-Gard)

B.	Aluminum Finish:

1.	Manufacturer's standard - clear anodized.

(other finishes available, please contact Duo-Gard)

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive translucent insulated daylighting.  Notify Architect or Interior Designer of conditions that would adversely affect installation or subsequent utilization of daylighting.  Do not proceed with installation until unsatisfactory conditions are corrected.

3.2	PREPARATION

A.	Ensure supports to receive translucent insulated daylighting are clean, flat, level, plumb, square, accurately aligned, and correctly located.

3.3	INSTALLATION

A.	Install translucent interior wall in accordance with manufacturer's instructions at locations indicated on the drawings.

B.	Install translucent interior wall level, plumb, square, accurately aligned, correctly located, and without warp.

C.	Anchor wall panels securely in place to supports.  Use attachment methods permitting adjustment for construction tolerances, irregularities, alignment, and expansion and contraction. Do not penetrate glazing with fasteners.

D.	Install wall panels including flashing, fasteners, hardware, gaskets, joint sealants, and glazing materials required for a complete and tight installation.

E.	Sheet Metal Flashing:  Install sheet metal flashing as specified elsewhere in specifications.

F.	Joint Sealants:  Install joint sealants as specified elsewhere in specifications.

G.	Repair minor damages to metal finish or glazing in accordance with manufacturer's instructions and as approved by Architect or Interior Designer.  Remove and replace damaged components that cannot be successfully repaired as determined by Architect or Interior Designer.

3.4 	CLEANING

A.	Clean wall panels in accordance with manufacturer's instructions.

B.	Clean inside and outside of wall panels immediately after installation and after joint sealants have cured.

C.	Remove temporary protective coverings at time of installation (interior) and after installation is complete (exterior).

D.	Remove excess joint sealant in accordance with sealant manufacturer’s instructions.

E.	Do not use harsh cleaning materials or methods that would damage metal finish or glazing.

3.5	PROTECTION

A.	Protect installed translucent wall from damage during construction.

B.	Remove and replace damaged wall panels as determined by Architect or Interior Designer.

END OF SECTION